# PART 1 – GENERAL

  - Section Includes

    - Sprayed cellulose acoustical finish system. (09 83 16)

# Related Items

    - Clips, hangers, supports, sleeves and other attachments to spray bases are to be placed by other trades prior to the application of the acoustical finish system.

    - Ducts, piping, conduit or other suspended equipment shall not be positioned until after the application of the acoustical finish system.

    - Roof penetrations to be installed prior to application.

# Quality Assurance

    - Manufacturer must be in compliance with the 2009-2021 International Building Code.

    - Manufacturer must be ISO 9001:2015 Certified.

    - Manufacturer shall have a minimum 10-year successful performance history of producing and installation of spray-applied cellulose on similar projects

    - Applicator: Licensed by manufacturer.

    - Mock-up: Apply a 100 square foot representative sample to be reviewed by the Architect and/or Owner prior to proceeding. (expand to include field sample instructions)?

    - Material must be tested in accordance with ASTM E 1042 by a NVLAP accredited testing laboratory.

# Submittals

    - Submit product data that the product meets or exceeds the following specified requirements.

      - Bond strength shall be greater than 100 psf per ASTM E 736.

      - Product shall be Class 1 Class A per ASTM E 84.

      - Non-corrosive per ASTM C 1149

      - Comply with 2015 IBC Section 803.12 stability requirements for interior finishes.

      - Product shall be Cradle to Cradle® Certified v.3.1 or higher to a minimum certification level of Bronze

      - Product shall be UL GREENGUARD Gold Certified

      - Product must have a publicly available Health Product Declaration (HPD) to 100 PPM

      - Product must have a third-party verified, publicly available, product-specific Environmental Product Declaration per ISO 14025

    - Manufacturer's written certification that product contains no asbestos, fiberglass or other man-made mineral fibers.

    - Copy of manufacturer’s ISO 9001:2015 Certification.

    - Minimum Fiber Recycled Content to be 75%.

    - Cannot contain any added Urea-Formaldehyde Resins.

# 1.06 Delivery, Storage and Handling

- Deliver in original, unopened containers bearing name of manufacturer and product identification.

- Store materials dry, off ground, and under cover.

- Protect liquid adhesive from freezing.

- Water to be potable.

# PART 2 – PRODUCTS

  - Acceptable Manufacturers

    - International Cellulose Corporation 12315 Robin Boulevard

Houston, Texas 77045

Phone: (713) 433-6701 or (800) 444-1252

Fax: (713) 433-2029

Website: www.spray-on.com Email: icc@saint-gobain.com

    - For approved applicators contact ICC at (800) 444-1252.

# Materials

    - SonaKrete® Acoustical Finish System

      - Color shall be from Manufacturer’s standard color chart or custom color as noted

      - Each bag must be labeled appropriately.

      - Each drum of adhesive must be labeled “SonaKrete® Adhesive”.

# PART 3 – EXECUTION

        - Examination

          - Examine surfaces and report unsatisfactory conditions in writing. Do not proceed until unsatisfactory conditions are corrected.

          - Verify surfaces to receive application of acoustical finish to determine if priming/sealing is required to insure bonding and/or to prevent discoloration caused by migratory stains.

# Preparation

          - Provide masking, drop cloths or other satisfactory coverings for materials/surfaces that are not to receive acoustical finish to protect from over-spray.

          - Coordinate installation of the acoustical finish system with work of other trades.

          - Prime surfaces as required by manufacturer’s instructions or as determined by examination.

          - General substrate preparation may include; but is not limited, to the following:

            - Gypsum/Drywall Substrates: A minimum level 3, preferably level 4 primed finish is recommended.

            - Drywall substrates are to be primed with a suitable drywall primer such as: Sherwin Williams® Multi-Purpose Water-Based Acrylic Alkyd.

            - The use of Quickset, Quicksand, Drywall Joint Compound is not acceptable.

            - Plaster substrates may require brown coat finish.

            - Consult the manufacturer for other substrates.

# Installation

          - Install acoustical finish system according to manufacturer’s recommendations.

          - Comply with local Building Code requirements.

          - Install the acoustical finish system to achieve an average NRC of 	.

| SonaKrete® Acoustical Finish System on Solid Backing: |  |
| --- | --- |
| Thickness: | NRC: |
| 0.25” | 0.50 |
| 0.375” | 0.55 |
| 0.50” | 0.65 |
| 0.75” | 0.75 |

Acoustical testing performed by NVLAP certified testing lab.

          - After achieving the required thickness and while still wet, float the material to match the approved field sample.

          - Cure the material with continuous natural or mechanical ventilation.

a.  Continuous ventilation must be maintained until the material has properly cured

          - Remove and dispose of over-spray.

# Protection

    - Protect finished installation under provision of Division 1.

END OF SECTION